TABLE OF CONTENTS

tv.lidrary/Brightness

tv.library/CloseTVWindow

tv.library/Contrast

tv.library/Hue

tv.library/I2CRead

tv.library/I2CTransfer

tv.library/I2CWrite

tv.library/OpenTVWindow

tv.library/ReadGPIO

tv.library/ReadTunerStatus

tv.library/ResetBt

tv.library/Saturation_U

tv.library/Saturation_V

tv.library/SetAudioMode

tv.library/SetSignalFormat

tv.library/SetTunerFreq

tv.library/SetVideoInput

tv.library/TVCardName

tv.library/VideoInputs

tv.library/Brightness

NAME

Brightness - sets brightness.

SYNOPSIS

Brightness(Bright)

Brightness( UBYTE )

D0

FUNCTION

This function sets brightness.

INPUTS

Bright - value from 0 to 255, default 128.

RESULT

NONE

SEE ALSO

Contrast(), Hue(), Saturation_U(), Saturation_V()

tv.library/CloseTVWindow

NAME

CloseTVWindow - closes TV window.

SYNOPSIS

CloseTVWindow(TVWindow)

CloseTVWindow( ULONG )

D0

FUNCTION

This function closes TV window.

INPUTS

TVWindow - pointer of the pointer of the window structure (in TV window mode)

or pointer of the internal tv.library structure (in backdrop mode).

RESULT

NONE

SEE ALSO

OpenTVWindow

tv.library/Contrast

NAME

Contrast - sets contrast.

SYNOPSIS

Contrast(Contrast)

Contrast( UBYTE )

D0

FUNCTION

This function sets contrast.

INPUTS

Contrast - value from 0 to 255, default 108.

RESULT

NONE

SEE ALSO

Brightness(), Hue(), Saturation_U(), Saturation_V()

tv.library/Hue

NAME

Hue - sets hue.

SYNOPSIS

Hue(Hue)

Hue( UBYTE )

D0

FUNCTION

This function sets hue.

INPUTS

Hue - value from 0 to 255, default 128.

RESULT

NONE

SEE ALSO

Brightness(), Contrast(), Saturation_U(), Saturation_V()

tv.library/I2CRead

NAME

I2CRead - reads byte from I2C bus.

SYNOPSIS

I2CRead (Address)

I2CRead ( UBYTE )

D0

FUNCTION

This function reads one byte from the I2C bus.

INPUTS

Address - address of chip connected to the I2C bus.

7 bits device address is set on bits 1-7, bit 0 always = 0.

RESULT

Data    - data from the I2C bus or -1 if error.

SEE ALSO

I2CWrite()

tv.library/I2CTransfer

NAME

I2CTransfer - software I2C transfers.

SYNOPSIS

I2CTransfer (NumMsg, PtrMsg)

I2CTransfer ( UWORD , ULONG )

D0      A0

FUNCTION

This function creates software transfers on I2C bus.

Table of i2c_Message structures describes type of transfers

that will be generated on I2C bus.

The start and stop transactions are automatically generated.

The restart transaction is not generated on start next i2c_message,

if I2CF_NORESTART flag is set (see tv.h and i2c bus specification)

Each i2c_Message can create only read or write transmission.

e.g.:

1) read UBYTE data from register for UWORD subaddress -

struct i2c_Messages MsgTab[]={{ devaddr,0,2,&subaddress},{devaddr,I2CF_READ,1,&data}};

I2CTransfer (2, &MsgTab);

INPUTS

NumMsg - number of i2c_Messges in table.

PtrMsg - pointer of table of i2c_Message structures.

RESULT

Status    - 0 if successful, -1 if error.

SEE ALSO

I2CRead(), I2CWrite

tv.library/I2CWrite

NAME

I2CWrite - writs bytes to the I2C bus.

SYNOPSIS

I2CWrite (Address, Data)

I2CWrite ( UBYTE , UWORD )

D0      D1

FUNCTION

This function writes bytes to the I2C bus.

INPUTS

Address - address of the chip connected to the I2C bus

7 bits of the device address is set on bits 1-7.

Bit 0 of the address describes number of bytes that will be sent

to the I2C bus.

Bit 0 = 0 - only one byte will be sent, otherwise two bytes.

Data    - bits 0-7 - byte1, bits 8-15 - byte2

RESULT

Status  - 0 if successful, -1 if error.

SEE ALSO

I2CRead()

tv.library/OpenTVWindow

NAME

OpenTVWindow - opens TV window.

SYNOPSIS

OpenTVWindow(Scaling,Cropping,WindowName)

OpenTVWindow( ULONG, ULONG, ULONG )

D0     D1     A0

FUNCTION

This function opens TV window. Window is opened in overlay mode.

Set TVB_BACKDROP bit in scaling parameter for tv screen

on workbench backdrop.

INPUTS

Scaling - value 1000 for full frame size (100%).

Cropping - value 1000 for full frame size (100%),

value 938 for regular TV screen.

WindowName - pointer of window name.

RESULT

TVWindow - pointer of the pointer of the window structure (in TV window mode)

or pointer of the internal tv.library structure (in backdrop mode)

or NULL.

SEE ALSO

CloseTVWindow

tv.library/ReadGPIO

NAME

ReadGPIO - reads GPIO port

SYNOPSIS

ULONG  ReadGPIO

FUNCTION

This function reads the GPIO port of Bt878 chip.

Some TV cards have connect to GPIO port IR reciver.

INPUTS

NONE

RESULT

Result - GPIO[0..24]

SEE ALSO

tv.library/ReadTunerStatus

NAME

ReadTunerStatus - reads tuner status.

SYNOPSIS

ReadTunerStatus()

FUNCTION

This function read tuner status byte.

For more information about byte status, see tuners documentations.

INPUTS

NONE

RESULT

Status - byte status.

SEE ALSO

tv.library/ResetBt

NAME

ResetBt - resets tv card chipset.

SYNOPSIS

ResetBt()

FUNCTION

This function resets tv cards chipset.

About results this function see tv chipsets documentation.

INPUTS

NONE

RESULT

NONE

SEE ALSO

tv.library/Saturation_U

NAME

Saturation_U - sets saturation of U component video signal.

SYNOPSIS

Saturation_U(Sat_U)

Saturation_U( UBYTE )

D0

FUNCTION

This function sets saturation of U component video signal.

INPUTS

Saturation_U - value from 0 to 255, default 90.

RESULT

NONE

SEE ALSO

Brightness(), Contrast(), Hue(), Saturation_V()

tv.library/Saturation_V

NAME

Saturation_V - sets saturation of V component video signal.

SYNOPSIS

Saturation_V(Sat_V)

Saturation_V( UBYTE )

D0

FUNCTION

This function sets saturation of V component video signal.

INPUTS

Saturation_V - value from 0 to 255, default 90.

RESULT

NONE

SEE ALSO

Brightness(), Contrast(), Hue(), Saturation_U()

tv.library/SetAudioMode

NAME

SetAudioMode - sets audio mode.

SYNOPSIS

SetAudioMode(AudioMode)

SetAudioMode( ULONG )

D0

FUNCTION

This function sets audio mode (tuner signal, radio signal, mute, ...)

Some modes are not implemented in some cards.

INPUTS

AudioMode - for values see tv.h.

RESULT

NONE

SEE ALSO

tv.library/SetSignalFormat

NAME

SetSignalFormat - sets signal format.

SYNOPSIS

SetSignalFormat(Format)

SetSignalFormat( UBYTE )

D0

FUNCTION

This function sets video signal format.

INPUTS

Format - for values see tv.h.

RESULT

NONE

SEE ALSO

tv.library/SetTunerFreq

NAME

SetTunerFreq - sets tuner frequency.

SYNOPSIS

SetTunerFreq(Pll, Flags)

SetTunerFreq( UWORD, UBYTE )

D0     D1

FUNCTION

This function sets tuner frequency.

INPUTS

Pll - frequency in MHz * 16, if frequency step is 62.50 kHz,

- frequency in MHz * 32, if frequency step is 32.25 kHz,

- frequency in MHz * 20, if frequency step is 50.00 kHz,

Flags - see tv.h file

RESULT

NONE

SEE ALSO

tv.library/SetVideoInput

NAME

SetVideoInput - sets video input.

SYNOPSIS

SetVideoInput(VideoInput)

SetVideoInput( UBYTE )

D0

FUNCTION

This function sets video signal input.

INPUTS

VideoInputs - value from 0 to 3 (to VideoInputs()).

usualy value 0 for tuner, 1 for external input

or camera. Set TVF_SVHS flag for SVHS input (see tv.h file).

RESULT

NONE

SEE ALSO

VideoInputs()

tv.library/TVCardName

NAME

TVCardName - returns tv card name.

SYNOPSIS

TVCardName()

FUNCTION

This function returns pointer of tv card name.

INPUTS

NONE

RESULT

Name - pointer of chars, NULL terminated.

SEE ALSO

tv.library/VideoInputs

NAME

VideoInputs - returns number of video signal inputs.

SYNOPSIS

VideoInputs()

FUNCTION

This function returns number of video signal inputs.

INPUTS

NONE

RESULT

Inputs - number of video signal inputs.

SEE ALSO